| ## Name | ## NOISECON |
| --- | --- |
| Author | Erhard Wielandt, Institute of Geophysics, University of Stuttgart, Richard-Wagner-Strasse 44, D - 70184 Stuttgart, E-mail: e.wielandt@t-online.de |
| Version | October 2001 |

This program accepts seismic noise specifications in all kind of standard and non-standard units and compares them to the USGS New Low Noise Model NLNM (Jon Peterson 1993, USGS Open File Report 93-322).

The program is interactive and self-explaining. It will ask you some multiple-choice questions about your noise data, then propose the correct physical units, and upon confirmation will ask for data, to be entered as a pair (frequency, amplitude) or whatever is appropriate. Note that the Fortran and Basic versions require the pair to be separated by a comma while the C version requires a space as a separator. The program will stop when the pair 0, 0 is entered.

For each data point entered, the program will write on the screen:

LNM:		the noise level of the NLNM in the units of your data

LNM pd:	the noise level of the NLNM in decibels re. (1 m/s2 )2 / Hz

your pd:	your data converted to decibels re. (1 m/s2 )2 / Hz

LNM amp:	Noise amplitudes of the NLNM in dB re.1 m/s2 rms amplitude in 1/6 decade, which is nearly the same as 1 m/s2 average peak amplitude in 1/3 octave

your amp:	your data converted to dB re.1 m/s2 rms amplitude in 1/6 decade

diff:		the level of your data relative to the NLNM in decibels

##### Download

Download source code and executable from

http://www.software-for-seismometry.de/