Type; # BBVS-60. Manufacturer; # Beijing GangZhen Mechanical 6 Electronic Technology Co. Ltd. Address; No. 20 Jinxing Rd, Daxing district, Beijing 102628, China. Phone (Fax); ++86-10-60212319 (-37). E-mail; geodevice@sina.com. Homepage; http://www.geodevice.cn/. Category; # Force-balance broadband, three component. Flat response; Velocity 16.7 mHz (60 s) to 50 Hz. Resolution: NLNM; 16.7 mHz to 8 Hz. within 10 dB of NLNM; 8 mHz to 20 Hz. within 20 dB of NLNM; 5 mHz to 50 Hz. Operating range; ±10 mm/s. Generator constant; 2000 Vs/m. Output; differential, ±10 V per line. Weight; 12 kg. Size; 24 cm dia., 27 cm high. Power; 9 to 18 volts, 1.1 watts. Manual control; Mass lock. Remote control; centering. Remote diagnostic; Mass position. Accessories; Tools, thermal insulation. Typical installation; All purpose. Data Sheet; http://www.geodevice.cn/en/productInfo.aspx?n= 20090921143217421562. Remarks; This instrument uses an astatic leaf-spring suspension like the Streckeisen seismometers..