Type; # CMG-3ESP Compact. Manufacturer; # Guralp Systems Limited. Address; 3 Midas House, Calleva Park, Aldermaston, Reading RG7 8EA, UK. Phone; ++44 118 9819056. E-mail; sales@guralp.com. Homepage; www.guralp.com. Category; # Force-balance BB, three-component. Flat response; velocity 33 mHz (30 s) to 50 Hz (others available). Resolution: NLNM; 40 mHz to 16 Hz (manufacturer). within 10 dB of NLNM. within 20 dB of NLNM. Operating range; ±10 mm/s. Generator constant; 2000 Vs /m (others available). Output; differential, ±10 V per line. Weight; 8 kg. Size; 17 cm dia., 19 cm high. Power; 10 to 36 volts, 0.6 watts (low-power version available). Manual control; -. Remote control; mass lock, centering, calibration. Remote diagnostic; mass position. Accessories; -. Typical installation; all purpose.. Data sheet; http://www.guralp.com/products/3ESPC/. Remarks; Guralp offers this seismometer with a wide variety of responses..