Type; # CMG-3ESP. Manufacturer; # Guralp Systems Limited. Address; 3 Midas House, Calleva Park, Aldermaston, Reading RG7 8EA, UK. Phone; ++44 118 9819056. E-mail; sales@guralp.com. Homepage; www.guralp.com. Category; # Force-balance BB, three-component. Flat response; velocity 33 mHz (30 s) to 50 Hz, others available. Resolution: NLNM; 25 mHz to 16 Hz (manufacturer). within 10 dB of NLNM. within 20 dB of NLNM. Operating range; ±10 mm/s. Generator constant; 2000 Vs/m, others available. Output; differential, ±10 V per line. Weight; 9 kg. Size; 17 cm dia., 32 cm high. Power; 10 to 30 volts, 0.6 watts. Manual control; mass lock. Remote control; centering, mass lock optional. Remote diagnostic; mass position. Accessories; control and breakout box. Typical installation; all purpose. Submersible version is available. Data sheet; http://www.guralp.com/products/3ESP/. Remarks.