Type; # CMG-3T, CMG3-TB (borehole). Manufacturer; # Guralp Systems Limited. Address; 3 Midas House, Calleva Park, Aldermaston, Reading RG7 8EA, UK. Phone; ++44 118 9819056. E-mail; sales@guralp.com. Homepage; www.guralp.com. Category; # Force-balance VBB, three-component. Flat response; velocity 8.33 mHz (120 s) to 50 Hz (others available). Resolution: NLNM; 30 mHz to 18 Hz*. within 10 dB of NLNM; 5 mHz to 30 Hz*. within 20 dB of NLNM; 3 mHz to 50 Hz*. Operating range; ±13 mm/s. Generator constant; 1500 Vs/m (others available). Output; differential, ±10 V per line. Weight; 14 kg. Size; 17 cm dia., 34 cm high. Power; 10 to 30 volts, 0.75 watts. Manual control; -. Remote control; mass lock, centering, calibration. Remote diagnostic; mass position. Accessories; control and breakout box. Typical installation; observatory, vault, and field use. Data sheet; http://www.guralp.com/products/3T/. Remarks; Tight thermal shielding recommended. Until recently the CMG-3T was one of the two most widely used broadband seismometers (the other one being the STS-2). The noise data are from the borehole version CMG-3TB.. * based on Ringler and Hutt 2010. Resolution above 10 Hz is extrapolated; data from the borehole version.