Type; # Episensor ES-T. Manufacturer; # Kinemetrics Inc.. Address; 222 Vista Avenue, Pasadena, CA 91107, USA. Phone; ++1 626 795 2220. E-mail; sales@kmi.com. Homepage; www.kinemetrics.com. Category; # Force-balance broadband accelerometer. Flat response; acceleration DC to 200 Hz. Resolution: NLNM; not applicable (this is a strong-motion instrument). within 10 dB of NLNM; n.a.. within 20 dB of NLNM; n.a.. Operating range; user selectable, ±0.25 g to ±4 g. Generator constant; user selectable, order of 1 V s2 / m. Output; ±2.5 volt single-ended to ±20 volt differential. Weight; 2 kg. Size; 13 cm dia., 6 cm high. Power; ±12 V or single 12 V, 0.15 to 0.4 W (depending on option). Manual control; Centering; set gain. Remote control; -. Remote diagnostic; -. Accessories; -. Typical installation; Anywhere: Strong-motion monitoring, seismic zonation. Data sheet; http://www.kinemetrics.com/p-87-EpiSensor-ES-T.aspx. Remarks; This is a modern and popular strong-motion sensor. Its large dynamic range overlaps considerably with that of high-sensitivity seismometers, making it possible to record microearthquakes and teleseisms with this instrument..