Type; # OYO-Geospace GS-1. Manufacturer; # OYO Geospace. Address; 7007 Pinemont, Houston, Texas 77040,USA. Phone; ++1-713-986-4444 Fax: -8723. E-mail; sales@oyogeospace.com. Homepage; http://www.oyogeospace.com/technologies. Category; ## Passive electromagnetic single-component short-period. Flat response; Velocity 1 Hz to >100 Hz (2 Hz version is available). Resolution: NLNM; 0.11 to 15 Hz with low-noise preamplifier*. within 10 dB of NLNM; 0.07 to 50 Hz with low-noise preamplifier*. within 20 dB of NLNM; 0.05 to 150 Hz with low-noise preamplifier*. Operating range; large, limited by preamplifier. Generator constant; 275 Vs/m with 4550 Ohms coil. output; differential, unlimited. Weight; 2 kg. Moving mass is 0.7 kg.. Size; 8 cm dia. , 16 cm high. Power; -. Manual control; no mass lock required. Short signal coil and tilt the instrument for transportation.. Remote control; -. Remote diagnostic; -. Accessories; -. Typical installation; temporary installations, field work. Data sheet; http://www.geospacelp.com/index.php?id=169. Remarks; A three-component package is available as “Seis-Monitor”. * based on circuit analysis with op-amp LT1007 as a preamplifier. Low-noise model may be inaccurate above 10 Hz.