Type; # JCZ-1. Manufacturer; # Wuhan Liquan Seismic Observation Technology Co. Ltd. Address; No. 40 Hongshan Celu, Wuhan, Hubei, China. Phone and Fax; ++86-27-87864644. E-mail; actseis@gmail.com. Homepage; www.whlqtech.com. Category; # Force-Balance VBB, three component. Flat response; Velocity 2.8 mHz (360 s) to 10 Hz. Resolution: NLNM; 5 mHz (200 s) to 10 Hz. within 10 dB of NLNM; 1 mHz (1000 s) to 20 Hz. within 20 dB of NLNM. Operating range; ±10 mm/s (velocity), ±1 mm/s2 (VLP acceleration). Generator constant; 2000 Vs/m (velocity), 20000 Vs2/m (VLP acceleration). Output; differential, ±10 V per line. Weight; 30 kg. Size; 51 * 57 cm, 49 cm high. Power; 9 – 18 V, 30 W. Manual control. Remote control; Mass lock, centering. Remote diagnostic; Mass position (acceleration output). Accessories; TCU and RCU boxes. Typical installation; Observatory. Data Sheet; No English data sheet available at the time of writing. Remarks; Inner temperature stabilized with a thermostat.