Type; # KS-1. Manufacturer; # Geotech Instruments LLC. Address; 10755 Sanden Drive, Dallas, Texas 75238-1336, USA. Phone; (001) 214 221 0000. E-mail; info@geoinstr.com. Homepage; www.geoinstr.com/. Category; # Force-balance VBB, three-component. Flat response; velocity 3 mHz (330 s) to 5 Hz. Resolution: NLNM; 10 mHz to 5 Hz*. within 10 dB of NLNM; 4 mHz to 9 Hz*. within 20 dB of NLNM; 0.3 mHz to 12 Hz*. Operating range; ±8 mm/s. Generator constant; 2400 Vs/m. Output; differential, ±10 V per line. Weight; 43 kg. Size; 27 cm dia., 34 cm high. Power; 24 V, 2.4 W. Manual control; -. Remote control; mass lock, centering, calibration. Remote diagnostic; mass position. Accessories; -. Typical use; Observatory. Data sheet; http://www.geoinstr.com/ds-ks1.pdf. Remarks; This is the same system as the KS54000 borehole seismometer but packaged in two separate units (mechanical sensors and electronics) for surface installation.. * based on Ringler and Hutt 2010.