Type; # KS-2000. Manufacturer; # Geotech Instruments LLC. Address; 10755 Sanden Drive, Dallas, Texas 75238-1336, USA. Phone; (001) 214 221 0000. E-mail; info@geoinstr.com. Homepage; www.geoinstr.com/. Category; # Force-balance, three-component, BB seismometer. Flat response; velocity 83 mHz to 50 Hz. Resolution: NLNM; 50 mHz to 15 Hz*. within 10 dB of NLNM; 35 mHz to 50 Hz*. within 20 dB of NLNM; 15 mHz to >100 Hz*. Operating range; ±10 mm/s. Generator constant; 2000 Vs/m. Output; differential, ±10 V per line. Weight; 11 kg. Size; 19 cm dia., 29 cm high. Power; 9 to 36 V, no current specified. Manual control; mass lock, centering (standard). Remote control; mass lock, centering (M and borehole models), calibration. Remote diagnostic; mass position. Accessories. Typical installation; Vault, field deployment. Data sheet; http://www.geoinstr.com/ds-ks2000m.pdf. Remarks; Based on the KS-54000 but smaller and about 20 dB noisier at low frequencies. Standard broadband response. Borehole and short-period versions are available.. * based on Ringler and Hutt 2010. Resolution above 10 Hz is extrapolated.