Type; # Le-3d-5s. Manufacturer; # Lennartz Electronic. Address; Bismarckstrasse 136, D-72072 Tuebingen, Germany. Phone; ++49 7071 93550. E-mail; info@lennartz-electronic.de. Homepage; www.lennartz-electronic.de. Category; # Active medium-period, three-component. Flat response; velocity 0.2 to 40 Hz. Resolution: NLNM; 0.15 to 1 Hz (microseismic peak). within 10 dB of NLNM; 0.13 to 15 Hz. within 20 dB of NLNM; 0.1 to 40 Hz. Operating range; ±13 mm/s below 4.5 Hz, decreasing at higher freq.. Generator constant; 400 Vs /m. Output; ±5 V single-ended. Weight; 6.5 kg. Size; 195 mm dia., 165 mm high. Power; 12 V, 0.12 W. Manual control; no mass lock required. Remote control; calibration. Remote diagnostic; -. Accessories; -. Typical installation; monitoring networks, field deployment. Data sheet; http://www.lennartz-electronic.de/MamboV4.5.2/index.php? option=com_content&task=view&id=72&Itemid=55. Remarks; A small and rugged three-component, medium-period, active seismometer. The sensor is a commercial 2 Hz geophone whose response is electronically extended by a negative damping resistance (a form of negative feedback). Not as sensitive as larger seismometers but good enough where ground noise is not extremely low..