Type; # PMD BB 603. Manufacturer; # Precision Measurement Devices. Address; 105F W. Dudleytown Rd., Bloomfield, CT 06002, USA. Phone; ++1 860 242 8177. E-mail; sales@pmdsci.com. Homepage; http://www.pmdsci.com/. Category; # Molecular-electronic BB three-component, with force feedback. Flat response; velocity 16.7 mHz (60 s) to 50 Hz. Resolution: NLNM; 0.05 Hz (20 s) to 5 Hz (manufacturer). within 10 dB of NLNM. within 20 dB of NLNM. Operating range; ±10 mm/s at low frequencies. Generator constant; 2000 Vs/m (other values optional). Output; differential, ±10 V per line. Weight; 11 kg. Size; 20 cm dia., 22 cm high. Power; 9 – 30 volt, 35 mW; low-power version (15 mW) available. Manual control; No mass lock or centering required. Remote control; Calibration, gain setting over serial port (optional). Remote diagnostic; -. Accessories; -. Typical installation; battery-powered stations, harsh environments. borehole and OBS versions available.. Data sheet; http://www.pmdsci.com/pdfs/BB603.pdf. Remarks; An unconventional sensor. The horizontals are essentially circular-tube tiltmeters where the flow of an electrolyte through a platinum mesh is measured using the MET effect (molecular-electronic transfer, also used in the “solion” transducer). The vertical sensor supports a column of electrolyte in a vertical tube with a membrane and a spring. These sensors have extremely low power consumption, are very rugged, need not be locked or leveled; they simply start working after power-on. Their performance in other respects is however inferior to force-balance sensors..