Type; # REFTEK 151-120 “Observer”. Manufacturer; # Refraction Technology, Inc.. Address; 1600, 10th Street, Suite A, Plano TX 75074, USA. Phone; ++1 214 440 1265. E-mail; Info @ reftek.com. Homepage; http://www.reftek.com/. Category; # Force-balance VBB, three components. Flat response; Velocity 8.3 mHz (120 s) to 50 Hz. Resolution: NLNM; 8 mHz to 15 Hz*. within 10 dB of NLNM; 3 mHz to 30 Hz*. within 20 dB of NLNM; 0.4 mHz to 50 Hz*. Operating range; ±10 mm/s. Generator constant; 2000 Vs/m. Output; differential, ±10 V per line. Weight; 12 kg. Size; 24 cm dia., 27 cm high. Power; 9 to 18 volts, 1.3 watts. Manual control; mass lock. Remote control; centering. Remote diagnostic; mass position. Accessories. Typical installation; observatory, vault. Data Sheet; http://www.reftek.com/pdf/151-120.pdf. Remarks; A very good VBB sensor, in the free-mode band comparable to KS-1 and CMG-3TB according to Ringler & Hutt, 2010.. * based on Ringler and Hutt 2010. Resolution above 10 Hz is extrapolated.