Type; # Trillium Compact. Manufacturer; # Nanometrics Inc.. Address; 250 Herzberg Road, Kanata, Ontario, Canada K2K 2A1. Phone; ++1 613 592 6776. E-mail; info@nanometrics.ca. Homepage; http://www.nanometrics.ca/. Category; # Miniature Force-balance BB, symmetric-triaxial. Flat response; velocity 8.3 mHz (120 s) to 100 Hz. Resolution: NLNM; 50 mHz to 8 Hz*. within 10 dB of NLNM; 45 mHz to 20 Hz*. within 20 dB of NLNM; 30 mHz to 50 Hz*. Operating range; ±26 mm/s. Generator constant; 750 Vs/m. Output; differential, ±10 V per line. Weight; comparable to a 1 Hz geophone. Size; comparable to a 1 Hz geophone. Power; 9 to 36 volt, 0.16 watt. Manual control; none: no mass lock or centering required. Remote control; RS-232 compatible integrated web server, UVW selection. Remote diagnostic; mass positions, temperature, serial number, poles and zeros. Accessories; -. Typical installation; temporary installations, field work. Data sheet; http://www.nanometrics.ca/products/trillium-compact. Remarks; OBS and all-terrain versions with internal self-leveling gimbal are available.. * based on Ringler and Hutt 2010 and the manufacturer