Type; # Trillium 120 P / PA. Manufacturer; # Nanometrics Inc.. Address; 250 Herzberg Road, Kanata, Ontario, Canada K2K 2A1. Phone; ++1 613 592 6776. E-mail; info@nanometrics.ca. Homepage; http://www.nanometrics.ca/. Category; # Force-balance VBB, symmetric-triaxial. Flat response; velocity 83 mHz (120 s) to 145 Hz. Resolution: NLNM; 25 mHz to 12 Hz*. within 10 dB of NLNM; 4 mHz to 20 Hz*. within 20 dB of NLNM; <0.3 mHz to 50 Hz*. Operating range; ± 16 mm/s. Generator constant; 1200 Vs/m. Output; differential, ±10 V per line. Weight; 7.5 kg. Size; 21 cm dia., 22 cm high. Power; 9 to 36 volt, 0.62 watt. Manual control; centering (120P). No mass lock required. Motorized mass centering (120PA).. Remote control; centering, UVW/XYZ mode, short/long period mode, firmware updates. Remote diagnostic; mass positions, temperature, serial number, status. Accessories; -. Typical installation; all purpose. Data sheet; http://www.nanometrics.ca/products/trillium-120-ppa. Remarks; Large temperature range without re-centering (±45o). A posthole version with remote leveling is available.. * based on Ringler and Hutt 2010 and the manufacturer