Type; # Trillium 240. Manufacturer; # Nanometrics Inc.. Address; 250 Herzberg Road, Kanata, Ontario, Canada K2K 2A1. Phone; ++1 613 592 6776. E-mail; info@nanometrics.ca. Homepage; http://www.nanometrics.ca/. Category; # Force-balance VBB, symmetric-triaxial. Flat response; velocity 41.7 mHz (240 s) to 35 Hz. Resolution: NLNM; 10 mHz to 10 Hz*. within 10 dB of NLNM; 0.3 mHz to 20 Hz*. within 20 dB of NLNM; 0.1 mHz to 35 Hz*. Operating range; ± 16 mm/s. Generator constant; 1200 Vs/m. Output; differential, ±10 volts per line. Weight; 14 kg. Size; 25 cm dia., 29 cm high. Power; 9 to 36 volt, 0.65 watt. Manual control; none: no mass lock required. Remote control; integrated web browser with many control options. Remote diagnostic; Mass positions, temperature, serial number, status, calibration parameters. Accessories; prefabricated heat shield. Typical installation; observatory, vault.. Data sheet; http://www.nanometrics.ca/products/trillium-240. Remarks; An excellent very-broad-band seismometer, comparable to the STS2 in its performance. May be slightly better than the STS2 at the lowest frequencies when magnetically shielded. Has a modern digital control interface.. * based on Ringler and Hutt 2010 and the manufacturer