| ## Name | ## SINFIT |
| --- | --- |
| Author | Erhard Wielandt, Institute of Geophysics, University of Stuttgart, Richard-Wagner-Strasse 44, D - 70184 Stuttgart, E-mail: e.wielandt@t-online.de |
| Version | October 2001 |

This program fits a sine-wave to the first signal and then a sine-wave with the same frequency to the second signal. The frequency of the sine-wave is determined from the first signal alone, which is supposed to be an undisturbed reference signal. A start value for the frequency can either be entered or automatically determined from the number of zero crossings of the first signal.

Test signals for this (and other) programs can be generated with TESTSIG. Both programs are interactive and self-explaining.

The structure of the data files is as follows:

- one header line, arbitrary (will be echoed but not evaluated)

- additional header or comment lines whose firs  character is %

- one line containing the number of samples, the FORTRAN format in

which they are listed, and the sampling interval. These three

entries must be in the FORTRAN format (i10,a20,f10.x).

- data in the specified format

- You may easily change this file structure in the source code.

##### Download

Download source code, executable, and test data from

http://www.software-for-seismometry.de/